Specifications

DESIGNED FOR ENERGY-EFFICIENT LIGHTING SOLUTIONS FOR MAXIMUM VISUAL COMFORT AND OPTIMUM LIGHT DISTRIBUTION IN MULTI-USE INDOOR ENVIRONMENTS

The luminaire shall consist of an acrylic PMMA and aluminum body and an opal diffuser. A flicker free non-dimming driver is standard equipped.

It is available in 600mm*600mm and 1200mm*600mm. It is ideal for exposed grid suspended ceilings. A surface mounted, mild steel powder coated enclosure is optional for surface mounting requirements.

The luminaire shall bear the IEC 60598 safety mark and shall have an ingress protection of IP20.

Use of high efficiency LED’s >90 lumens per watt and CRI >80.

The standard LED’s colour temperature provided shall be 4000K, 5000K or 6500K.

The LED PANEL luminaire shall operate LED light source of up to 65W, in operating temperatures (Ta) between -100C up to 50˚C, without reducing the LED lifetime of 50 000hrs, at a lumen depreciation of not more than 30% (L70).

The driver shall operate at a power factor of >0,90 and the total harmonic distortion levels are < 20% and do not cause interference on the electrical network.

For ease of installation, a quick wiring connection is provided.

| - Updated by | - Approved by | - Document Revision / Date |
| --- | --- | --- |
| - Nadja Smith | - Pieter van Aardt | - 20/07/2023 |