LEDlume XP

Technical Product Specifications

Table of contents

1.	General Specifications	3

2.	Photometry	5

3.	Mechanics	8

4.	Electrical Characteristics	9

5.	Power Supply	10

6.	Environment	10

- General Specifications

| Product Description | Side entry luminaire designed for optimized residential road, roads & motorways, car parks, pedestrian and perimeter lighting. The luminaire consists of a spigot base compartment and hinged LED engine top casting. This hinged design allows the easy installation of the LED engine and the stainless steel latches allows for easy closure achieving the IP 66 water ingress protection of the electronic components. The design can operate LEDs of up to 265W. The mains terminal connector enclosed inside a cavity on the spigot base for improved water ingress. The luminaire shall bear the SANS 60598-2-3 safety mark. The luminaire shall have a degree of protection that complies with SANS 60598-2-3: LED compartment: IP 66 The IP rating is supported by a certified SABS test report. The LED engine, consisting of the LED light source and the power supply, can be easily replaced or upgraded. To maximize the reliability of the LED’s, the photometrical engine is completely sealed to IP 66. This ensures that the photometric performance is maintained over time. Electronic temperature monitoring prevents overheating of LED’s and power supply. The spigot base casting easily slides onto a 42mm outreach and secures with 2 M8 grub screws. The two stainless steel latches make for easy tool free maintenance. It has a blade connector that allows for electrical disconnection as soon as the luminaire is opened. Manufactured from marine grade aluminium allows for corrosion resistance and optimal thermal management. A silicon sponge gasket fixed into a groove seals the spigot base casting to the LED engine casting to IP 66. The high-impact clear glass protector allows for impact resistance of up to IK 09 with the option of Polycarbonate protector with an impact resistance of IK 10. All LED’s have individual lenses for optimal light distribution for improved light performance. Additional anti-glare louvres and back light control is available. All control gear components are removable and bear the relevant SABS mark. All internal wiring are Teflon® coated to prevent damage by possible abrasion. All screws, bolts and metal parts are stainless steel or non-corrosive material. Mains connections by means of a suitable screw terminal block with a wire clamping contact. The luminaire power factor corrected to a minimum of 0,9. The IP 66 LED housing compartment optimizes the thermal operating environment around the LEDs enabling the long useful lifetime (100 000hrs, L95B10). With the introduction of LED technology, dimming options are much easier and further maximize energy savings. |
| --- | --- |
| Main applications | - Road lighting & Motorways - Squares & Pedestrian area lighting - Security perimeter lighting - Parking areas - Bike & Pedestrian Paths |
| Recommended mounting height | 4m - 30m |

- Photometry

| Light source | LED |
| --- | --- |
| LED | 2mm2 LED |
| Light color | 4000K (Neutral white 740) 3000K (Warm white 730) – optional 5700K (Cool white 757) – optional |
| Color rendering (Ra) | ≥70 |
| Power consumption 12W 17W 23W 35W 23W 32W 45W 54W 69W 33W 47W 66W 96W 43W 61W 88W 100W 55W 78W 111W 65W 93W 111W 131W 84W 122W 147W 151W 217W 179W 257W 209W 254W 239W 265W | Nominal Lumen package 1905lm 2576lm 3395lm 4501lm 3810lm 5109lm 6790lm 7864lm 9002lm 5714lm 7729lm 10281lm 13650lm 7560lm 10131lm 13452lm 14969lm 9226lm 12555lm 16814lm 11071lm 15066lm 17622lm 20177lm 14761lm 20088lm 23496lm 25110lm 33629lm 30132lm 40355lm 35154lm 41117lm 40176lm 43584lm |
| Optics | Asymmetric |
| Photometry: | - Please note: Optic 5304-W only applicable to 40 LED, 80 LED, 120 LED and 160 LED versions |

| Description | LED Current | LED | Nominal Lumen* | Lifetime Residual Flux @ tq 25°C |
| --- | --- | --- | --- | --- |
|  |  |  |  | @100 000h |
| LEDlume XP 1 12W | 350mA | 10 | 1905 | 95% |
| LEDlume XP 1 17W | 500mA | 10 | 2576 |  |
| LEDlume XP 1 23W | 700mA | 10 | 3395 |  |
| LEDlume XP 1 35W | 1000mA | 10 | 4501 |  |
| LEDlume XP 1 23W | 350mA | 20 | 3810 |  |
| LEDlume XP 1 32W | 500mA | 20 | 5109 |  |
| LEDlume XP 1 45W | 700mA | 20 | 6790 |  |
| LEDlume XP 1 54W | 850mA | 20 | 7864 |  |
| LEDlume XP 1 69W | 1000mA | 20 | 9002 |  |
| LEDlume XP 2 33W | 350mA | 30 | 5714 | 95% |
| LEDlume XP 2 47W | 500mA | 30 | 7729 |  |
| LEDlume XP 2 66W | 700mA | 30 | 10281 |  |
| LEDlume XP 2 96W | 1000mA | 30 | 13650 |  |
| LEDlume XP 2 43W | 350mA | 40 | 7560 |  |
| LEDlume XP 2 61W | 500mA | 40 | 10131 |  |
| LEDlume XP 2 88W LEDlume XP 2 100W | 700mA 800mA | 40 40 | 13452 14969 |  |
| LEDlume XP 3 55W | 350mA | 50 | 9226 |  |
| LEDlume XP 3 78W | 500mA | 50 | 12555 | 95% |
| LEDlume XP 3 111W | 700mA | 50 | 16814 |  |
| LEDLUME XP 3 65w LEDLUME XP 3 93W LEDLUME XP 3 111W LEDLUME XP 3 131W | 350mA 500mA 600mA 700mA | 60 60 60 60 | 11071 15066 17622 20177 |  |
| LEDLUME XP 3 84w LEDLUME XP 3 122W LEDLUME XP 3 147W | 350mA 500mA 600mA | 80 80 80 | 14761 20088 23496 |  |
| LEDLUME XP 4 151W LEDLUME XP 4 217W | 500mA 700mA | 100 100 | 25110 33629 | 95% |
| LEDLUME XP 4 179W LEDLUME XP 4 257W | 500mA 700mA | 120 120 | 30132 40355 |  |
| LEDLUME XP 4 209W LEDLUME XP 4 254W | 500mA 600mA | 140 140 | 35154 41117 |  |
| LEDLUME XP 4 239W LEDLUME XP 4 265W | 500mA 550mA | 160 160 | 40176 43584 |  |

(*) The type of LED used is subject to change due to the ongoing rapid progress taking place in LED technology

- Mechanics

| Electronic control Gear | Constant Current LED Driver IP 66 |
| --- | --- |
| Materials and finishing | Spigot base casting – Marine grade aluminium (EN 1706 AC-44300) LED Engine – Marine grade aluminium Protector – High-impact clear glass |
| Coating | Unpainted aluminium RAL colours on request |
| Installation | Side Entry Pole Mount |
| Fixing | Side Entry Pole Mount – 2 x M8 grub screws |
| Dimensions (LxWxH) (mm) | XP 1: 444 x 244 x 104* XP 2: 535 x 244 x 107* XP 3: 618 x 345 x 111* XP 4: 386 x 835 x 120* *STD version |
| Weight (with gear) | XP 1: 4kg XP 2: 4.5kg XP 3: 9kg XP 4: 13kg |
| Access | Yes |
| Aerodynamic resistance (CxS): | XP 1: 0.046m2 XP 2: 0.057m2 XP 3: 0.072m2 XP 4: 0.088m2 |

- Electrical Characteristics

| Line Voltage | 230VAC |
| --- | --- |
| Mains voltage Tolerance (AC) | 198-264V |
| Line frequency | 50Hz |
| Electrical Safety Class (IEC) | Class I or II |
| Surge protection | 10kV/10kA 20kV/20kA - optional |
| Lighting control | Optional - NEMA 3-pin socket only - - NEMA 7-pin socket only - NEMA with Royce Thomson DLS - NEMA with Spectrum DLS - NEMA 3-pin with dummy link - NEMA 7-pin with plug-in 20kV surge - Schréder EXEDRA CM - DIM (Optidim) - DIM Profile (1-5) |

- Power Supply

| Power Factor | ≥0.95 |
| --- | --- |
| Removable | Yes |
| Thermal Safety | Yes |

- Environment

| Storage temperature Operating temperature | -20°C up to 35oC -35°C up to +40°C |
| --- | --- |
| Enclosure Tightness | IP 66 |
| Enclosure Mechanical Withstand Impact | Glass: XP 1 & XP 2: IK 08 XP 3 & XP 4: IK 09 Polycarbonate: IK 10 |
| Enclosure Mechanical Withstand Vibrations | Modified IEC 60068-2-6 |

| - Updated by | - Approved by | - Document Revision / Date |
| --- | --- | --- |
| - Nadja Smith | - Pieter Van Aardt | - 07/11/2023 |

- For specific Tq, Ta and lifetime data, contact BEKA Schréder